DISASSEMBLER USER'S MANUAL

for

D51  V2.6   8051/8052

DZ80 V2.1      Z80

D48  V2.1 8035/8048/8041

Jeffery L. Post

22726 Benner Ave,

Torrance, CA 90505

E-mail:   ThePosts@soca.com

March,  1998

MSDOS is a trademark of Microsoft Corporation

PCDOS is a trademark of International Business Machines

TABLE OF CONTENTS

Introduction .................................................. page 1

Quick start ................................................... page 3

Command line options .......................................... page 4

A option .................................................. page 4

B option .................................................. page 5

D option .................................................. page 5

H option .................................................. page 5

S option .................................................. page 5

X option (D51 and DZ80 only)............................... page 6

Control file directives ....................................... page 7

A directive ............................................... page 8

B directive ............................................... page 8

C directive ............................................... page 8

F directive (D51 only)..................................... page 9

I directive ............................................... page 9

K directive (D51 only)..................................... page 9

L directive ............................................... page 10

M directive (D51 only)..................................... page 10

R directive (D51 only)..................................... page 10

S directive ............................................... page 10

T directive ............................................... page 11

W directive ............................................... page 12

Format of Output File ......................................... page 13

Background .................................................... page 15

Technical info ................................................ page 16

Summary ....................................................... page 20

8051 register maps ............................................ page 22

8051 bit addressable special function registers ............... page 23

8051 bit addressable memory locations ......................... page 23

Page 1

INTRODUCTION

Version 2 is a significant improvement over Version 1. Version 2

allows the user to specify areas of the code to be disassembled

as something other than executable code; ie: as ascii text, binary

data, address tables, etc (see the section on control file direct-

ives). You may specify such areas by means of an ascii text file

with the same name as the file to be disassembled, but with an

extension of CTL (for example: program.ctl controls the disassembly

of program.hex or program.bin). By means of the control file, you

can, by an iterative process, get a progressively more meaningful

disassembly by modifying the control file based on examination of

the output of previous passes of the disassembler. You can also

specify names for labels and symbols in the control file, resulting

in a much more readable source file. A file named GENERIC.CTL is

provided as a means to get the user started; just copy GENERIC.CTL

to FILE.CTL, where FILE is the name of the file to be disassembled,

and make appropriate changes. Labels and symbols may have as many

characters as will fit in one line of the control file, but some cross-

assemblers (and especially cross linkers) will truncate any more

than some fixed number of characters. Consult your cross development

tools' documentation. Labels and symbols are not case sensitive.

Like Version 1, Version 2 still does not analyze the code; the user

must do so and control disassembly via the control file.

Unlike Version 1, which was distributed as a public domain program

without source code, Version 2 is being distributed with source code

under the GNU General Public License (see the file COPYING for

details). The author has benefited greatly from software distributed

under the GNU GPL and wishes to thank those who have contributed so

much by adding his own meager efforts to the large base of freely

distributable software. It is the author's contention that software

patents are an oxymoron; that although the United States Patent Office

recognizes that mathematical algorithms are not patentable, they are

too ignorant to recognize the fact that ANY software is expressable as

a mathematical algorithm. The author specifically refuses to acknowledge

the legitimacy of any software patents whatsoever. Copyrights and

trade secrets are, however, another matter.

In accordance with the spirit of the GNU General Public License, the

following notice is added to this documentation:

Page 2

This program is free software; you can redistribute it and/or modify

it under the terms of the GNU General Public License as published by

the Free Software Foundation; either version 2 of the License, or (at

your option) any later version.

This program is distributed in the hope that it will be useful, but

WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANT-

ABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU General

Public License for more details.

You should have received a copy of the GNU General Public License

along with this program; if not, write to the Free Software

Foundation, Inc., 675 Mass Ave, Cambridge, MA 02139, USA.

The file COPYING contains the full text of the GNU General Public

License, Version 2.

The quick start section of this document gives some minimal informa-

tion for users who are eager to try out the disassemblers. For best

use of the disassemblers, however, you should read the sections on

command line options and control file directives before proceeding.

The section entitled 'background' describes the purpose of a dis-

assembler, how a disassembler works (in basic terms), and the reasons

you might want to use one. This section is intended for novice users

but might be interesting to more advanced users.

The technical info section covers disassembly techniques and gives a

detailed description of the disassemblers and their source code. This

is intended for more advanced users who might want to modify or expand

them, or even create a disassembler for a different 8 bit processor.

Those who peruse this section, and the source code, will see that the

techniques employed are only applicable to processors that are limited

to 64K or less of program memory (most 8 bit processors).

Page 3

QUICK START

To disassemble a file called 'code.bin' or 'code.hex', type in (at

the DOS prompt):

copy generic.ctl code.ctl

to create a control file for the disassembly, and then type:

d51 code d        (or DZ80/D48)

A control file is not necessary for disassembly, but as will be seen,

it will provide for a much more complete disassembly.

If neither the 'b' option nor the 'h' option is specified, the

disassembler will

first search for a hex file and, if it is not found, then search for

a bin file. The disassembler can be forced to look for only one or the

other by specifying the (b)inary or (h)ex option on the command line:

d51 code b d		for the file code.bin, or

d51 code h d		for the file code.hex

D51 will produce a file named 'code.d51' containing the disassembly of

the 8051/8052 instructions in the original hex or binary file. It is

very useful on the first few disassemblies to use the 'd' option, which

adds a comment field to every line. The comment field contains the

hexadecimal address of each instruction, the hex data for each byte in

the instruction, and the ascii code for each byte of the instruction.

Example:

(opcode) (operands)	(address) (data)   (ascii)

orl	a,#42h		; 0df6   44 42      DB

This makes it easy to spot areas of ascii text or other nonexecutable

code, and then modify the control file to tell D51 that these areas

should be treated as some other kind of data on subsequent disassem-

blies. Please see the section on control file directives for specific

information.

DZ80 produces source files with an extension of '.z80' and D48 produces

source files with extensions of '.d48'.

By examining the output file and then modifying the control file based

on the information obtained, you can get a progressively better disas-

sembly of your program. Generally, about half a dozen iterations

of the disassembly are enough to produce a very readable source file

that can then be modified and reassembled.

Page 4

COMMAND LINE OPTIONS

Command line options modify the operation of the disassemblers.

They may be specified on the command line using either Unix style

options:

d51 -db filename

or MSDOS style options:

d51 /d /b filename

Options may be entered in a freeform fashion (in any order or format)

provided that any options preceeding the filename must begin with

either a dash (-) or a slash (/). For example:

d51 /db filename -x1000

d51 filename -d b x1000

d51 -d filename bx1000

all mean exactly the same thing and will cause the disassemblers to

behave in exactly the same way.

Command line options in Version 2 are:

A OPTION (ascii macro):

The A option tells the disassembler to change the db/defb pseudo

op to the ascii macro for areas defined as ascii text. This will

cause the disassembler to include a macro definition in the

disassembly output file that should expand to 'db' for the cross

assembler. To use this option, your cross assembler must be able

to handle macros, and in particular, D51's macro format. If used

in conjunction with the S option (see below), the S option must

come first on the command line.

There are some caveats regarding the ascii macro. The intent of

this macro is to make ascii text easier to find in the output file.

Once you're satisfied with the readability of the output file, you

may want to run the disassembler once more without the A option to

produce a file more digestible by your cross assembler.

Some cross assemblers treat certain characters within a macro as

special meta-characters. The Huntsville Microsystems HMA8051, for

example, regards the characters '#', '.' and '\' as special with-

in a macro. If these characters occur in ascii text using the

ascii macro of D51, the assembler will do strange and not so

wonderful things with them. If this happens, just do not use the

A option (by default, it is off).

A problem that will be common with all cross assemblers is that

the disassemblers do not currently treat the single quote (')

character differently than it does other characters within

Page 5

ascii text.

Since the disassembler delineates ascii text (whether using the

A option or not) with the single quote character, its presence

in ascii text areas will cause problems. The only solutions to

this right now are to manually edit the output file of the

disassembler or to define the byte of the single quote as a binary

byte in the control file (see the B control file directive).

B OPTION (binary file):

The B option forces the disassembler to read a binary file

(extention '.bin') instead of an Intel hex file (extention .hex).

If neither the B nor the H option are specified, the disassembler

will first search for a file with an extention of .hex, and if not

found, will then search for a binary file with an extention of .bin.

D OPTION (data in comment field):

The D option tells the disassembler to include a comment field

in each disassembled line. The comment field will contain the

hexadecimal address of each instruction, the hex data for each

byte in the instruction, and the ascii code for each byte of the

instruction. Example:

(opcode) (operands)	(address) (data)   (ascii)

orl	a,#42h		; 0df6   44 42      DB

This makes it easy to spot areas of ascii text or other non-

executable code, and then modify the control file to tell the

disassembler that these areas should be treated as some other

kind of data on subsequent disassemblies. Please see the section

on control file directives for specific information.

H OPTION (hexadecimal file):

The H option forces the disassembler to read an Intel hex file

(extention '.hex') instead of a binary file (extention '.bin').

If neither the B nor the H option are specified, the disassembler

will first search for a file with an extention of '.hex', and if

not found, will then search for a binary file with an extention

of '.bin'.

S OPTION (string pseudo op):

This option tells the disassembler to generate the pseudo ops

'defb' and 'defw' instead of 'db' and 'dw' for those cross

assemblers that may require this. Most cross assemblers understand

'db' and 'dw', so these are the defaults for the disassembler. Use

this option for cross assemblers that do not recognize 'db' and

'dw'. If used in conjunction with the A option, the S option must

preceed the A option on the command line.

Page 6

X OPTION (hexadecimal offset - D51 and DZ80 only):

This option causes the disassembler to add a hexadecimal offset

to the address of every location in the code file. This may be

useful for disassembling code that was read from the second (or

subsequent) rom of a set of roms. For example, suppose that

binary data was obtained from a set of 4K byte eproms (call them

rom1 and rom2). Then the first file should be disassembled with

the command line:

d51 -d rom1

and the second should be disassembled with the X option:

d51 -d rom2 x1000

The two files then can be combined into one and all references

within the disassembled code will be to the correct addresses.

Page 7

CONTROL FILE DIRECTIVES

Control directives tell the disassembler to treat specified areas

of the code to be disassembled as something other than executable

code. The user specifies control directives by editing (with any

text editor) a control file with the same name as the file to be

disassembled, but with a file extention of .ctl (eg: program.ctl

to control the disassembly of the file program.hex or program.bin).

A generic control file called 'generic.ctl' is provided to get the

user started. Copy 'generic.ctl' to 'file.ctl' where 'file' is the

name of the binary or Intel hex file to be disassembled, and modify

as required based on the output of the disassembler. Then run the

disassembler again to get a more readable disassembly of the code.

All directives consist of a character that specifies the type of data,

followed by a hexadecimal number specifying the value or range of

values of the data. In the case of a label or symbol directive, the

hexadecimal number is followed by ascii text defining the label or

symbol. See individual directives for examples. All directives must

be in the first column of the line. Directives may be complete words,

but only the first character is significant, ie:

Label	0	reset				and

looney	0	reset

will both generate the label "reset" for address 0. Note that they

are not case sensitive.

Everything following a semi-colon (;) is a comment and will be

ignored by the disassembler.

Label and symbol directives differ in that a label refers to an

address and a symbol refers to any 8 or 16 bit data. Therefore an

opcode such as

mov	r2,#data

will cause the disassembler to search only the symbol table for a

matching value whereas an opcode such as

mov	dptr,#data

or

ld	hl,data

will cause the disassembler to search the label table first, and

if no matching value is found, to then search the symbol table.

Single values are specified by entering just the value; ranges of

values are specified by a start and stop address separated by a dash

(-), or by a start address and count separated by a plus (+), ie:

t 1000		the data at address 1000H is ascii text.

t 1000-1010	the data from address 1000H to (and including)

address 1010H is ascii text.

t 1000+6	ascii text starting at 1000H for 6 bytes.

Page 8

Directives are processed in the order in which they are read from

the control file. Specifying an area as ascii data and then later

specifying it as data to be ignored (uninitialized data) will cause

that area to not be disassembled at all. Care must be taken in

specifying areas in the control file.

A DIRECTIVE (address data):

Specifies that the address contains a word value corresponding

to an address for which a label should be generated. For example,

a vector table may be located at address 0x1000 containing four

entries. The user can modify the control file by adding the

following entry:

a 1000-1007

This will cause the disassembler to generate the following

output lines:

dw	vec1

dw	vec2

dw	vec3

dw	vec4

assuming that the control file also contains entries for the

values found at those addresses to generate the labels vec1

through vec4. The addresses referenced will be flagged so

that labels will be generated in the output file at those

addresses.

B DIRECTIVE (byte binary data):

Specifies that the data is to be interpreted as 8 bit binary

data. For example:

b 1000-1007

tells the disassembler that the data from address 1000H to 1007H

should generate the line:

db	0,1,2,3,4,5,6,7

assuming that the data at address 1000H and up is 00H, 01H, etc.

C DIRECTIVE (code data):

Forces the disassembler to interpret the data as executable code.

This may be necessary because the disassembler skips over strings

of 00H or ffH bytes that occur since they are unlikely to be real

code. However, sometimes programmers insert several NOPs for timing

purposes.

Page 9

F DIRECTIVE (SFR label definition - D51 only):

Causes reference to an SFR (special function register) to

generate a label for the specified SFR address. This is useful

for naming SFR's for non-standard members of the 8051 family.

For example, suppose that some manufacturer designs a new 8051

variant and decides to use address efH as a new special function

register called 'NEWREG'. Then by adding:

f ef newreg

to the control file, D51 will disassemble the opcodes f5 ef as:

mov	newreg,a

instead of:

mov	0efh,a

SFR label names are limited to fifteen characters.

I DIRECTIVE (ignore data):

Tells the disassembler to ignore a range of addresses that may be

initialized by the input file. This is useful when the input file

is a binary file generated by an eprom programmer that dumps the

entire eprom space. The valid data from a 4K eprom might only be,

say, 3K in length. By adding

i c00-fff

to the control file, you tell the disassembler to not disassemble

the data from address 0c00H to 0fffH.

K DIRECTIVE (SFR bit definition - D51 only):

As with the F directive, you can give a name to a new SFR bit.

If a new variant of the 8051 family adds a new interrupt, for

instance, its enable bit in the IE register might be bit 6. By

adding:

k ae newint	(ae is the address of IE.6)

you define bit 6 of the interrupt enable register to be

ie.newint

Like SFR label names, SFR bit names are limited to fifteen characters.

Page 10

L DIRECTIVE (label):

Defines a label to the disassembler. Labels are generated in the

output disassembly file whenever a reference to the address is

found and the label exists in the label table. Suppose that

address 0000H contains the code 01 43 (ajmp 0043H). Then the entry

l 43 start

in the control file will cause D51 to disassemble this code as:

ajmp	start

rather than:

ajmp	X0043

The code beginning at address 43H will then have the label

'start' in the label field rather than the label 'X0043'. See

the entry for the S directive for an explanation of the diff-

erence between labels and symbols.

M DIRECTIVE (bit addressable memory label - D51 only):

Similar to the K directive, the M directive gives a name to a

bit addressable memory location. Example:

m 12	bitflag		(12h is the bit address of 22h.2)

would cause the bytes d2 12 to be disassembled as

setb	bitflag

instead of

setb	22h.2

As with the K directive, names are limited to fifteen characters.

R DIRECTIVE (register definition - D51 only):

This directive specifies a name for an 8051 register. Direct

accesses to registers will normally be disassembled as:

mov	rb2r5,a

in the case of register 5 of register bank 2. If the user

determines that register 5 of bank 2 is being used as a loop

counter, for example, he can give it the name loop by adding:

r 15 loop

to the control file, and the above code will be disassembled as:

mov	loop,a

The R directive can assign a name to any internal memory location

below the SFR address space.

S DIRECTIVE (symbol):

Defines a symbol to the disassembler. Symbols are output to the

disassembly file whenever the value is encountered in the input

file and the symbol exists in the symbol table. The user can

specify a symbol for a value by a line such as:

s 20 space

Page 11

Code which uses this value, such as:

mov	a,#20h

or

ld	a,#20h

will then be disassembled as:

mov	a,#space

or

ld	a,#space

Labels and symbols differ in that a label applies to an address,

whereas a symbol applies to any 8 or 16 bit data. For code which

can only refer to values, such as

mov	a,#20h

or

ld	c,#55h

only the symbol table will be searched for a matching value. For

code which could refer to either an address or a value, such as:

mov	dptr,#1234h

or

ld	hl,#1234h

the label table will be searched first, and only if no matching

value is found will the symbol table then be searched.

T DIRECTIVE (text [ascii] data):

Forces the data in the range specified to be disassembled as

text (ascii data). Thus the code: 48 69 20 74 68 65 72 65 at

address 1000H will be disassembled as:

db	'Hi there'

or

ascii	'Hi there'	(command line option A used)

by putting the line:

t 1000-1007

in the control file. Otherwise it would be interpreted as:

orl	a,r0		; 1000   48	     H

xrl	a,r1		; 1001   69	     i

jb	2eh.4,X106d	; 1002   20 74 68    th

xrl	a,72h		; 1005   65 72      er

xrl	a,t3		; 1007   65 ff      e.

which is unlikely to be what was intended by the original

programmer. Data specified by this directive is not checked

to verify that it really is ascii data--be careful.

Page 12

W DIRECTIVE (word binary data):

Specifies that the data is to be interpreted as 16 bit binary

data. For example:

w 1000-1007

tells the disassembler that the data from address 1000H to 1007H

should generate the lines:

dw	0

dw	1

dw	2

dw	3

assuming that the data at address 1000H and up is 00H, 00H,

00H, 01H, 00H, 02H, etc. This differs from the A (address)

directive in that the A directive also causes a reference

to be made to the indirect address. In other words, if the

data at address 1000H is 1234H, the A directive will cause

a label (X1234) to be put in the output file at address

1234H, whereas the W directive will not.

Page 13

FORMAT OF OUTPUT FILE

The output file for D51 (extention '.d51') contains 8051 instructions

and pseudo ops in a format that should be acceptable to any 8051

cross assembler. DZ80 outputs files with extention '.z80' and D48

outputs files with extention '.d48'. When assembled, the source file

should produce an Intel hex file or binary file that is identical to

the original. Identical, in this case, means that the code is the

same, not that it necessarily has the same format. The number of

bytes in each line of the hex file may differ from the original

depending on whether the original was produced by an assembler or

was read from an eprom.

Each output line is in the following format:

LABEL:	OPCODE	OPERANDS	; COMMENT

The label field will be generated only if the address of the code

is referenced by some other code (a call, for instance), or if the

user has specified a label for that address in the control file.

The comment field is only generated if the user specifies the D

option on the command line. It begins with a semicolon and is

followed by the hexadecimal address of the instruction, the hex

value of the byte(s) for the instruction, and finally by the ascii

representation of the instruction byte(s). For example:

X004f:	orl	p3,#0ffh	; 004f   43 b0 ff   C0.

label	opcode	operands	comment

Several things should be noted in this example. The label is 'X'

followed by the address of the instruction in four hex digits.

This type of label is created by the disassembler when some other

code references the address. If the user specifies a label name for

the address in the control file ('what' for example), the generated

code becomes:

what:	orl	p3,#0ffh	; 004f   43 b0 ff   C0.

If the user were to specify the symbol 'ones' for the value 0ffH,

the output would then become:

what:	orl	p3,#ones	; 004f   43 b0 ff   C0.

Also note the ascii representation of the instruction bytes. Bit

7 is forced to zero so that ascii with the high bit set can be

viewed (b0H in the example becomes 30H, which is ascii '0'). Any

code which is not printable ascii (a control code, for example)

is output as a period. This is the case for the byte 0ffH above.

Page 14

The comment field starts at a different position in the output

line depending on the type of data in the input file. For normal

executable code, the comment field starts in column 32; for hex

data it starts at column 49, and for ascii data it starts in

column 56. For many programs, this makes for a more readable

source file, but if there is a mix of data types in a short range

of addresses, this can lead to some strange looking formatting.

The user may want to correct this with a text editor in the final

disassembly before modification and reassembly.

Page 15

BACKGROUND

A disassembler is a program that reads machine code from a binary

or hexadecimal file and generates an assembly language source code

file. You may want to do this to recover source code which has been

lost, or to modify the code in equipment which uses the processor

or just for the educational value of finding out how something works.

THESE DISASSEMBLERS SHOULD NOT BE USED TO REVERSE ENGINEER A PRODUCT

WHERE DOING SO WOULD BE UNETHICAL (reverse engineering a competitor's

product, for example). By using any of these disassemblers, you

accept sole responsibility for determining whether or not their use

is appropriate. If in doubt, consult a lawyer (if you can tolerate

being near one).

The source file generated by the disassembler should be compatible

with a cross assembler that could then be used to create machine

code identical with the original (except for modifications the user

may wish to make).

The disassembler reads the entire program from the hex or binary file

into memory, then reads the control file to flag any data the user

specifies as something other than executable code. When this is done,

the disassembler scans the data array containing the code three times.

On the first pass, it looks for code such as calls or jumps that

reference other code in the array. When such code is found, the

referenced address is flagged so that a label can be output to the

source file in the second pass. The second pass creates the output

file and writes the assembly source code lines. See the section titled

'Technical Info' for a description of this process. The third pass

through the code generates equate statements at the end of the file

for anything that requires it. This includes labels and symbols

defined in the control file, addresses that were referenced but are

not the first byte of an instruction or are not within the program

space read from the input file, and miscellaneous data values for

which no symbol or label has been specified in the control file.

On the first attempt to disassemble your code, use the generic control

file GENERIC.CTL by copying it to CODE.CTL (where CODE is the name of

the binary or hex file to be disassembled). After examining the source

code generated by the disassembler, modify the control file with any

ascii text editor to force the disassembler to interpret such areas as

executable code (or tables of addresses or whatever). Then re-run the

disassembler for a more complete disassembly. Repeat this process as

many times as necessary to achieve an acceptable level of disassembly

(please see the documentation for control file directives).

Page 16

TECHNICAL INFO

Disassembly Techniques:

The disassemblers are table driven disassemblers. The value of each 8

bit opcode to be disassembled is used as an index into two tables. The

mnemonic table contains ascii text entries for the mnemonic for each

opcode. The option table ('opttbl') contains 8 bit flags which tell

the disassembler what type of opcode is being disassembled so that it

can generate proper operands.

The disassembler first analyzes the command line to get the name of the

file to be disassembled and any command line options. After finding and

opening the file, the program allocates two 64K arrays: one to hold the

program to be disassembled and one for disassembly control flags. Both

arrays are initialized with codes that indicate invalid data. The input

file is then read and the data stored in the program array. As the data

is stored, the corresponding byte in the control array is set to

indicate valid program data.

Once the program is in memory, the disassembler looks for a control

file with the same name as the program file (but with an extension of

.ctl). If a control file is found, it is read and the information is

used to modify the control array flags to indicate what type of data

is in the area of memory specified in the control file directives. The

control file can also specify label and symbol names, among other things.

In order to keep disassembly times reasonably short with large control

files, the symbol and label tables are sorted after the control file is

read, and a binary search is performed whenever a label or symbol needs

to be found. The sort used is a non-recursive in-place merge sort.

The advantages of this sort are:

1) Memory space for a secondary list is not required since

it is an in-place sort.

2) Worst case performance is O(nlogn).

3) Being non-recursive, it doesn't require a huge stack.

The sort is a modification of the non-recursive bottom-up merge sort

described in Algorithms in C, by Robert Sedgewick, Addison-Wesley, 1990

(pp. 168). The optimized binary search is derived from an invariant

binary search from Data Structures and Program Design in C, by Kruse,

Leung and Tondo, Prentice-Hall, 1991 (p 155). The other algorithms in

this program are entirely my own fault.

To allow large control files, Version 2 disassemblers have been

compiled using the 'compact' memory model (for 80x86 POS processors).

The disassembler then executes three passes over the program data. The

first pass looks for executable code that references some other address

(calls and jumps, for example.) The referenced address is then flagged

(in the control flag array) so that a label will be generated in the

output file during the second pass.

Page 17

The second pass does the real work of the disassembler. Pass 2 scans

the program array and checks to see if the current location contains

executable code or some other type of data (as specified by the

control array flags.) If the current location contains some type of

data that is not executable code, the control flags determine what

action will be taken. If the control flags specify binary byte or

ascii data, the data is temporarily stored in a buffer so that multiple

bytes can be output on one line, rather than having a separate line with

a DB pseudo op for each byte of data. Word and address data, however,

do generate a separate line for each entry.

If the current location contains executable code, the disassembler

outputs the mnemonic for the instruction (by using the opcode as an

index into the mnemonic table), and then uses the option table flags

indexed by the opcode to determine how to generate appropriate operands.

During this pass, D51 keeps track of any direct accesses to registers

and outputs equates for their names at the end of the pass.

The third and last pass does some clean up work--searching the array

for references to un-initialized data or references to addresses not

at the beginning of multi-byte opcodes so that equate statements can

be generated. This pass also outputs equate statements for the entries

in the symbol table.

Source Code:

The source code for the disassemblers is contained in five files

for each disassembler where 'xx' is '51', '48' or 'Z80':

Dxx.H		Header file defining constants, program flag

values and data structures.

Dxx.TBL		Tables for mnemonics, option flags, 8051 SFR

names, 8051 SFR bit names and register names.

Dxx.C		Main function that analyzes the command line,

finds and opens the disassembly file, allocates

memory for data arrays and calls the disassembly

functions in dxxp1.c and dxxp2.c.

DxxP1.C		Contains 'pass0', which is not really a pass but

reads the control file and modifies the flag array

accordingly, and 'pass1' which searches for

address references. This file also has routines

relating to the label and symbol tables.

DxxP2.C		Main disassembly routine that generates the output

file (pass 2). Also contains 'pass3' which generates

equates for unresolved references and for the

symbol table entries.

Each of the C files contains prototypes for the routines in the file.

If your compiler doesn't support prototypes, it's time to get a new

compiler.

Page 18

The header file (Dxx.H) defines constants for exit codes, program flags

and various limits. It also contains structure definitions for symbol

and label table entries, mnemonic table entries and sfr names. The file

Dxx.TBL contains the mnemonic table, the option flag table, and for D51,

the sfr name and bit name tables and the register name table.

Dxx.C is the main disassembler file. The function main() parses the

command line for user options. It then attempts to open the input file

(bin or hex) and allocates memory for program and flag data. The input

file is then read into the program data array.  Main then calls various

routines in other files to perform the disassembly. Dxx.C also has the

following routines:

atox()		Converts ascii hex to binary hex data.

getcode()	Extracts data from ascii hex file and puts it in

the 64K program array.

find_entry()	Searches for an entry in the symbol or label table.

get_smem()	Allocates memory for a new symbol or label entry

and links it into a singly linked list.

add_entry()	Adds a new entry into the symbol or label list.

DxxP1.C contains pass0(), which isn't really a disassembly pass, but

reads the control file (if any) and sets up the program flag array

according to the directives found in the control file. The label and

symbol tables are built from data found in the control file. Pass1()

scans through the program array and, if the program flags define the

data as executable code, determines if the code references a program

location (jumps, calls, etc). Any such references set the program

flags for the referenced address. This is necessary so that pass 2 can

generate labels for the referenced addresses. DxxP1.C also has routines

to dump accumulated data for db pseudo ops (ascii and binary).

DxxP2.C is the real workhorse of the disassembler. Pass2() generates

the output assembly source file by scanning through the program array

and disassembling the code based on the program flag array info. If

the program flag specifies that the data is code, the opcode byte is

used as an index into two arrays: the mnemonic table, which contains

ascii text for the opcode, and the option table, which has bit flags

to tell the disassembler what type of operands are involved. It is

this table driven feature that makes the disassemblers fast. This

technique, however, limits any disassembler based on it to processors

that have, at most, 64K of program space. It would not be applicable

to, for example, a 680x0 processor, which can have up to 4 Gbytes of

program space (unless you have a computer that can handle 4 Gbyte

arrays). If the program flags specify that the data is ascii or binary

data, the data is saved in a separate array so that many bytes can be

output with one db pseudo op. When pass 2 is complete, pass3() rescans

the data array looking for references that require equate pseudo ops.

Pass 3 also outputs equates for symbol and label table entries.

Further details of the disassembler's implementation can be found by

examing the source files.

Page 19

Caveats:

Considerable time has been spent in designing the code to produce an

intelligent disassembly of the data. If the user defines some area

as code (in the control file), it will be disassembled as code even

if it is a bunch of NOPs (which would normally be truncated). If the

user defines an area as ascii text and the high order bit of the

location is set, an attempt will be made to convert it to an ascii

form, ie: 0c5h will be disassembled as "'E'+80h" in the output file.

Despite the effort to disassemble in an intelligent manner, some

peculiar things may still happen. Occassionally the disassembler

will produce spurious labels or equates for locations that are not

referenced. Sometimes this is caused by ambiguities in the code

being disassembled. For example: a Z80 instruction such as

ld	hl,0dh

will produce a reference to address 0dh. It may be that the user has

defined a symbol for the value 0dh, but no label for that address.

In this case the disassembler will search the label table for the

value 0dh, and when it is not found, will search the symbol table.

If the user has defined 0dh as the symbol 'CR', then the output

becomes:

ld	hl,CR

but a reference has still been made to location 0dh and the label

'X000d:' will be put at that location even though it may not appear

anywhere else in the output file. This is not a problem for an

assembler since a defined label that is not referenced elsewhere in

the code will simply be ignored. A more serious problem would be if

the disassembler produces a source line such as:

ld	hl,X000d

and the label 'X000d' does not appear anywhere else in the output

file. This would cause an assembler error. I would appreciate being

informed of any problem that causes an assembler error.

Page 20

SUMMARY

Version 2 is being distributed under the GNU General Public License.

Complete source code for all the disassemblers MUST accompany any

distribution of this software. You are free to modify it in any way

you see fit, but you are NOT free to market it as a commercial product

(except for reasonable fees for media or other reproduction and/or

distribution costs). See the file COPYING for details.

Version 2 are disassemblers that allow the user to specify how code

should be disassembled via a control file that defines given areas of

the code as ascii text, binary data or other types of non-executable

code. By an iterative process of disassembly, control file modification,

and further disassembly, the user can obtain a very readable source file

that can then be modified and re-assembled by a cross assembler. Several

public domain cross assemblers are available.

I've encountered several problems when reassembling the output of D51.

Almost all are due to the cross assembler not recognizing SFRs of some

of the newer or more exotic 8051 variants. The D51 tables include

names for all SFRs that I'm aware of as of May 1995 (there are likely

some that I'm not aware of). Such errors might be corrected just by

defining a symbol for the SFR in the control file (see the F directive),

but this depends on whether your cross assembler will accept it as a

valid operand. If all else fails, replace the offending instruction

with a 'db' pseudo op and put the mnemonic and operand in a comment.

A similar problem is the assembler not being able to handle a bit

addressable SFR address; the only solution I see to this is to use a

'db' pseudo op. All other problems I've seen (not many) have been due

to disassembling data as code; this is easily corrected by modifying

the control file to specify the correct data type.

Version 2 disassemblers were compiled using Borland's Turbo C V2.0

(compact memory model), Borland C++ V3.1 and GCC running under Linux.

Note: As of 1998, time constraints prevent me from testing all the

above compiler/OS combinations. D51 is now tested only with Borland

C++ V3.1 under MSDOS. If you use another compiler or OS, you're on

your own.

Many thanks to Orris Dent for pointing out deficiencies, problems and

errors with D51, and for helpful suggestions for improvements. Orris

has been my de facto beta tester, and his suggestions have contributed

greatly to the quality of D51. (Of course, any remaining deficiencies

are entirely my own fault.) Dan Stover and others also contributed hex

files that uncovered unforseen problems. Thanks to all whose efforts

have made D51 a better product.

Page 21

Summary of command line options and control file directives

Command line options:

A - Change db/defb pseudo op to 'ascii' macro for text

B - Force read of binary file (.bin extention)

D - Add address and data info in comment field

H - Force read of Intel hex file (.hex extention)

S - Change db and dw pseudo ops to defb and defw

X - Add hexadecimal offset to all program locations

Control file directives:

A - Specifies that words in program space are addresses

B - Data is 8 bit binary

C - Program space is executable code

F - Specify SFR label

I - Ignore data (uninitialize program space)

K - Specify label for SFR bit

L - Define label

M - Specify label for bit addressable memory

R - Define register name

S - Define symbol

T - Specify ascii text

W - Data is 16 bit binary

Page 22

8051 REGISTER MAPS

Addresses of 8051 registers:

Register Bank   R0  R1  R2  R3  R4  R5  R6  R7

----------------------------------------------

0           00  01  02  03  04  05  06  07

1           08  09  0A  0B  0C  0D  0E  0F

2           10  11  12  13  14  15  16  17

3           18  19  1A  1B  1C  1D  1E  1F

Addresses of 8051 Special Function Registers:

Address Symbol Name

----------------------------

80   P0     Port 0

81   SP     Stack Pointer

82   DPL    Data Pointer (low byte)

83   DPH    Data Pointer (high byte)

87   PCON   Power Control

88   TCON   Timer/Counter Control

89   TMOD   Timer/Counter Mode

8A   TL0    Timer/Counter 0 (low byte)

8B   TL1    Timer/Counter 1 (low byte)

8C   TH0    Timer/Counter 0 (high byte)

8D   TH1    Timer/Counter 1 (high byte)

90   P1     Port 1

98   SCON   Serial Port Control

99   SBUF   Serial Data Buffer

A0   P2     Port 2

A8   IE     Interrupt Enable

B0   P3     Port 3

B8   IP     Interrupt Priority

D0   PSW    Program Status Word

E0   ACC    Accumulator

F0   B      B Register

Page 23

Bit Addressable Special Function Registers:

Address Symbol SFR bit Name

-------------------------------

80-87       P0.0-7  Port 0 Bits

88   IT0    TCON.0  External Interrupt 0 Type Control

89   IE0    TCON.1  External Interrupt 0 Edge Control

8A   IT1    TCON.2  External Interrupt 1 Type Control

8B   IE1    TCON.3  External Interrupt 1 Edge Control

8C   TR0    TCON.4  Timer 0 Run Control

8D   TF0    TCON.5  Timer 0 Overflow Flag

8E   TR1    TCON.6  Timer 1 Run Control

8F   TF1    TCON.7  Timer 1 Overflow Flag

90-97       P1.0-7  Port 1 Bits

98   RI     SCON.0  Serial Receive Interrupt Flag

99   TI     SCON.1  Serial Transmit Interrupt Flag

9A   RB8    SCON.2  Serial Receive Bit 8

9B   TB8    SCON.3  Serial Transmit Bit 8

9C   REN    SCON.4  Serial Receive Enable

9D   SM2    SCON.5  Serial Port Mode Specifier

9E   SM1    SCON.6  Serial Port Mode Specifier

9F   SM0    SCON.7  Serial Port Mode Specifier

A0-A7       P2.0-7  Port 2 Bits

A8   EX0    IE.0    Enable External Interrupt 0

A9   ET0    IE.1    Enable Timer 0 Interrupt

AA   EX1    IE.2    Enable External Interrupt 1

AB   ET1    IE.3    Enable Timer 1 Interrupt

AC   ES     IE.4    Enable Serial Port Interrupt

AF   EA     IE.7    Enable Interrupts

B0-B7       P3.0-7  Port 3 Bits

B8   PX0    IP.0    External Interrupt 0 Priority

B9   PT0    IP.1    Timer 0 Interrupt Priority

BA   PX1    IP.2    External Interrupt 1 Priority

BB   PT1    IP.3    Timer 1 Interrupt Priority

BC   PS     IP.4    Serial Port Interrupt Priority

D0   P      PSW.0   Parity

D2   OV     PSW.2   Overflow Flag

D3   RS0    PSW.3   Register Bank Select Bit 0

D4   RS1    PSW.4   Register Bank Select Bit 1

D5   F0     PSW.5   Flag 0 (general purpose flag)

D6   AC     PSW.6   Auxiliary Carry Flag

D7   CY     PSW.7   Carry Flag

E0-E7       ACC.0-7 Accumulator Bits

F0-F7       B.0-7   B Register Bits

Addresses of Bit Addressable Memory Locations:

Address		Memory location

-------------------------------

00-07	20.0 - 20.7

08-0f	21.0 - 21.7

10-17	22.0 - 22.7

18-1f	23.0 - 23.7

20-27	24.0 - 24.7

28-2f	25.0 - 25.7

30-37	26.0 - 26.7

38-3f	27.0 - 27.7

40-47	28.0 - 28.7

48-4f	29.0 - 29.7

50-57	2a.0 - 2a.7

58-5f	2b.0 - 2b.7

60-67	2c.0 - 2c.7

68-6f	2d.0 - 2d.7

70-77	2e.0 - 2e.7

78-7f	2f.0 - 2f.7