Generating Sine waves with 8051Fx's PWM Generator.

Kevin W. Clark

INTRODUCTION

The 8051Fx family's Programmable Counter Array (PCA) includes

provisions for generating Pulse Width Modulator (PWM) outputs.  With

suitable filtering, these PWM outputs can generate an analog voltage.

For a constant voltage, you can just set and forget.  It is also

possible to generate a varying waveform by changing the PWM value with

each pulse.

This application note details the hardware and software needed to

generate sine waves and even more complex waveforms with the PWM

outputs.  The sine waves can be of any  arbitrary frequency, and

several sine waves of different frequencies can be generated together.

Some of the possible uses for this technique include annunciator or

alarm tone generation, DTMF tone generation for phone equipment, music

in three-part harmony, multi-phase motor drive, quadrature sine waves

for signal processing, or precision sine-wave generation

SOFTWARE

This technique is based on a frequency synthesis technique known as

Direct Digital Synthesis (DDS).  In DDS, a counter is used to step

through a Sine look-up table, which is then fed into a DAC.  However,

the counter can count by any arbitrary increment, including fractional,

and is called a phase accumulator.

In our program, the phase accumulator is a 16 bit variable called

PHASE_ACC.  On each interrupt, it has the value in PHASE_INC added to

it, and the upper byte of the result is used as an index into the 256

byte sine table.  This value is then output to the PWM channel, which

is updated on the next full cycle.  The value can be optionally

multiplied by a gain factor for amplitude control, for instance to

achieve high-band pre-emphasis in a DTMF generator.

These variables can be regarded as having an integer portion and a

fractional portion thusly: iiiiiiii.ffffffff, where the integer portion

is in "degrees" of 1/256 of a cycle.  This integer portion is used to

index into the sine table.

The sine table was generated by the program SINTAB.BAS which produced

the file SINTAB.A51, which was then merged into the source file.

Generating the periodic interrupt is a bit tricky, since the PCA

resources used for PWM generation conflict with those used for the

interrupt logic.  Fortunately, another PCA channel can be used to

generate the interrupt.  This is done by setting the other channel ( we

used # 3 ) to compare mode to interrupt on count nn00, where nn is the

value in CCAP3H and is incremented on each interrupt.  This causes the

next interrupt to occur 256 cycles later.

HARDWARE

To yield a sine wave, the PWM's pulse output must be lowpass filtered.

A simple R-C filter is generally not adequate for audio applications,

due to its gradual roll-off, so an active filter of order 4 or higher

is recommended.  Figure 1 shows one possible second order filter

circuit.  Two of these, cascaded, generally give adequate performance.

Another possible filter would be a monolithic switched-cap filter such

as one of the Maxim MAX290 family of 8th-order lowpass filters, or the

Intel 2912A switched-cap CODEC filter.

Generating sine waves of greater than about 1/4 of the sample frequency

is not practical, since large low-frequency modulation components

result, which are very difficult to filter out.  Therefore the corner

frequency of the filter is set at 1/4 to 1/5 of the sample frequency.

Due to the quasi-bidirectional nature of the PWM output port pins,

either the filter should have a high input impedance, or the PWM output

should be buffered first.

The DTMF program assumes a certain hardware configuration for the

keypad or pushbutton switches, and would have to be changed to fit the

actual hardware used.

This technique doesn't require an 80C51Fx with PWM, by the way.  An

8-bit DAC added to any MCS-51 family part could be used with the

interrupt being provided by one of the timers, with very little change

in the code.

SPECIFICATIONS

The highest PWM frequency is Fxtal/1024, by setting PCA clock source at

Fxtal/4, so, for a 16MHz crystal, the PWM frequency can be as high as

15625Hz.  This can generate sines up to about 3KHz, which is adequate

for telephony or other audio applications.  For generating low

frequency sines, the PCA clock source can be Fxtal/12 or timer 2

overflow.  These lower frequencies also reduce processor overhead in

the interrupt routine.

With a 16MHz crystal, an interrupt occurs every 64uS.  The interrupt

routine takes 26.5uS to calculate a single value, for about 41%

processor utilization.  Much of this time is spent in normal interrupt

overhead, and it is possible to calculate up to four sine waves on each

interrupt.  Higher crystal frequencies can generate higher sine

frequencies, but the percentage of processor time remains constant.

It is also possible to generate up to 3 sine waves on one channel, with

amplitude control for each one.

Some performance results are as follows:

Total Microseconds per interrupt = 64uS at 16.000 MHz

Time spent in interrupt

PWM   simplest single sine generator	 		42.3% of total

DTMF  two tones without amplitude control		53.7%

DTMF  with amplitude control for each tone      	74.3%

3PART with amplitude control for each voice,		89.2%

all on one channel

4WAVE each on a separate channel, fixed amplitude.	70.9%

EXAMPLES

Several example files are included.  These are:

PWM.A51     	Single Sine wave generator.

DTMF.A51	Two tone generator.

3PART.A51	Three tones, with amplitude control.

4WAVE.A51	Four sine waves, each on a different channel.

SINTAB.BAS	Program to generate full amplitude table.

SINTAB2.BAS	Program to generate half amplitude table.

SINTAB3.BAS	Program to generate 1/3 amplitude table.

FIGURES

=========================

+------+--------+         for Fc of 3KHz:

|      |        |         R1 =  33K

C1     |  |\    |         R2 =  33K

|      +--|-\   |         C1 =  .002uF

|         |  >--+---o     C2 =  .001uF

o--R1--+--R2--+--|+/             U1 =  1/2  LM6482 CMOS OP-AMP

|  |/

|    U1

C2

|

|

=

Figure 1.  Active low-pass filter.

U1 is an LM6482 or similar CMOS op-amp with rail-to-rail input and

output range on a single +5v power supply.  This will allow a sine wave

output of 0.0 to 5.0v.  For different corner frequencies, scale the

resistors and/or capacitors accordingly.

=========================

REFERENCES

INTEL Embedded Microcontroller handbook, 270645